TAKTFLOW Continuous Assumption Validation Pipeline Every layer of automotive development carries forward assumptions about the layer below. This pipeline makes those assumptions continuously testable — from HARA to hardware. FIND IT IN WEEK 2 Not week 40 when the cost is 50× higher.; EVERY LAYER VERIFIED SIL → PIL → HIL on every commit, fully traced.; ONE TRACE CHAIN Requirement ID → test → result → evidence..

# 1. The Core Insight

Automotive software development fails at the seams between layers — not because engineers are wrong, but because every layer is written by people making assumptions about the layer below that nobody has yet run on hardware.

The V-model is designed to catch this. In practice it catches it too late — at the verification stage for that layer, which is weeks or months after the assumption was written, when correction costs have compounded through every layer above it.

The assumption chain runs from the highest-level safety analysis all the way down to physics:

HARA (OEM) assumes → safe state reachable in 10ms │ ↓ derives Technical Safety Concept assumes → diagnostic coverage 97% achievable │ ↓ derives Software Safety Requirements assumes → watchdog granularity 1ms on target OS │ ↓ derives BSW Configuration assumes → FreeRTOS tick + scheduler sufficient │ ↓ derives Firmware Implementation assumes → HAL interrupt latency negligible │ ↓ runs on Hardware reality → 23ms / 91% / 5ms / insufficient / 340μs ↑ where is this discovered? ─────────────────────────────────────┘ V-model answer: at hardware integration test, week 36–40 Taktflow answer: at first HIL run, week 2

This is not only about HARA. Every layer in the V-model carries assumptions forward:

| Layer | Assumption Carried Forward | Taktflow Validator |
| --- | --- | --- |
| HARA / Safety Goals | Safe state reachable within Xms. Fault detectable. ASIL assignment based on estimated controllability. | HIL fault injection — measures actual reaction time against safety goal |
| Technical Safety Concept | Diagnostic coverage percentage achievable. Fault reaction time bound. Safe state architecture sufficient. | SIL + PIL — measures coverage, validates fault reaction chain end-to-end |
| SW Safety Requirements | OS tick rate meets watchdog granularity. Scheduler priority sufficient for safety tasks. Stack depth adequate. | PIL on target — runs safety tasks on real RTOS, measures actual timing |
| Interface Specification | Signal encoding correct. Message cycle time compatible. E2E profile sufficient for error detection rate. | SIL restbus — sends malformed messages, validates E2E detection |
| BSW Configuration | CanIf/Com/PduR routing correct. ComSignal bit positions match DBC. RTE port mapping complete. | ARXML strict validation + SIL — catches mismatches before compile |
| Firmware Implementation | Algorithm behaves correctly under boundary conditions. Safe state FSM covers all fault combinations. | SIL unit tests — exercises all FSM branches, all boundary values |
| Integration | ECU-to-ECU timing compatible. No CAN bus overload. Network management convergence within spec. | HIL restbus — 7-ECU live bus, measures actual bus load and timing |

# 2. The Pipeline Flow

Each layer of the V-model has a corresponding pipeline stage that runs automatically on every commit. The validator at each stage is chosen to surface assumption deviations at the earliest possible moment, before they propagate upward.

| L0 | Requirements & Safety Goals ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written Safe state reachable in Xms. ASIL D. Fault detected within diagnostic interval. | Reality (example) Reaction time exceeds target. Coverage below ASIL D threshold. Fault masked by another failure. | Validator HIL fault injection + traceability report. Req ID → test → measured result. | Artifact produced coverage_report.html — REQ-ID mapped to measured timing, PASS/FAIL per xIL level. |

↓ DBC + ARXML generated → ARXML strict validation CI gate (autosar-data) → 0 warnings required

| L1 | System & Interface Specification ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written Signal encoding correct. E2E profile sufficient. Message timing compatible across all ECUs. | Reality (example) Bit position off by 1. E2E CRC misses burst errors. Timing budget exceeded at 7-ECU integration. | Validator SIL restbus — injects malformed messages, timing violations, E2E error patterns. | Artifact produced JUnit XML — each interface assumption has a corresponding test case with measured result. |

↓ BSW configs generated from ARXML → arxmlgen → CanIf_Cfg.c Com_Cfg.c PduR_Cfg.c RTE headers

| L2 | BSW Configuration ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written CanIf routing correct. ComSignal positions match DBC. PduR paths complete. RTE ports wired. | Reality (example) Signal byte order inverted. PduR routing table missing one path. RTE port left unconnected. | Validator ARXML round-trip validation + SIL compile — catches mismatches before any hardware needed. | Artifact produced armodel delta report — schema compliance, reference integrity, round-trip identity confirmed. |

↓ Compile → GCC ARM / TI CGT → ELF → Docker SIL or OpenOCD flash to PIL target

| L3 | SIL — Software in the Loop ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written Algorithm correct under all boundary conditions. FSM covers all fault state combinations. | Reality (example) Safe state FSM misses double-fault case. Boundary value at 0xFF causes overflow. Stack depth exceeded. | Validator pytest --target=sil. GCC + Docker. Same test code that runs at PIL and HIL. MISRA/gcov checked. | Artifact produced JUnit XML + lcov coverage report — MC/DC evidence for ASIL D structural coverage requirement. |

↓ Flash to real silicon → OpenOCD → STM32H743 / TMS570LS3137 → PIL transport via semihosting

| L4 | PIL — Processor in the Loop ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written FreeRTOS tick rate meets watchdog granularity. Interrupt latency within budget. Compiler optimisation safe. | Reality (example) Optimised build changes timing by 8ms. IRQ priority inversion starves safety task. Stack overflow at runtime. | Validator pytest --target=pil. Same test cases as SIL, now running on target silicon, real RTOS, real compiler. | Artifact produced JUnit XML — SIL vs PIL delta immediately visible. Timing regression caught before hardware integration. |

↓ Live CAN bus → PCAN-USB FD → 7-ECU mixed HIL → Korad power sim → relay fault injection

| L5 | HIL — Hardware in the Loop ↓ assumption carried forward |  |  |  |
| --- | --- | --- | --- | --- |
|  | Assumption written ECU-to-ECU timing compatible. Bus load within spec. Safe state propagates correctly across all nodes. | Reality (example) Gateway adds 35ms latency not in spec. Bus load 94% under fault storm. Safe state loop not terminated. | Validator pytest --target=hil. hil_restbus.py + fault injection. Power-cycle scenarios. 7-ECU live integration. | Artifact produced JUnit XML + .asc CAN log — full measured evidence. Imports into CANoe, Polarion, Jenkins. |

↓ merge_xil_results.py → unified HTML report → REQ-ID → SIL PASS / PIL PASS / HIL PASS

VERIFIED FIRMWARE + FULL TRACEABILITY EVIDENCE Every requirement traced to a test. Every test run at SIL, PIL, and HIL. Every result timestamped and committed.

# 3. The Cost of Late Discovery

The fundamental economic argument: finding an assumption deviation early is cheap. Finding it late is catastrophic. The pipeline is an investment in early discovery.

| When Discovered | What You Find | Typical Cost to Correct |
| --- | --- | --- |
| Week 2 — SIL | Algorithm assumption wrong. FSM missing a state. Boundary value overflow. | €500 — fix the code, re-run CI |
| Week 3 — PIL | OS timing assumption wrong. Interrupt priority inversion. Compiler artefact. | €1,000 — adjust RTOS config, re-run |
| Week 4 — HIL | Interface assumption wrong. Signal timing incompatible between two ECUs. | €3,000 — BSW rework, integration re-test |
| Week 12 — System integration | Safety goal assumption wrong. Reaction time 2× over budget. | €50,000 — safety concept revision, full re-test campaign |
| Week 30 — Validation | ASIL assignment wrong. Coverage insufficient. Diagnostic monitor gap. | €200,000 — safety case rework, OEM negotiation, schedule slip |
| Post-SOP — Field | Safe state unreachable under specific fault combination. | €500,000–€5M — recall, field fix, liability |

The pipeline cost is fixed: 4 weeks to stand up per project, ~2 days/month to maintain. The cost of one week-30 discovery exceeds the pipeline cost for 10 years of operation.

# 4. What Runs on Every Commit

The pipeline is not a milestone activity. It runs automatically on every code push and reports within minutes. This is the operational difference from the traditional V-model — verification is continuous, not periodic.

git push origin feature/safe-state-rework [1/6] ARXML validation ............ autosar-data strict → PASS (0 warnings) armodel round-trip ........... delta = 6 nodes → PASS (threshold ≤20) [2/6] BSW codegen ................. arxmlgen → CanIf_Cfg.c Com_Cfg.c MISRA check ................ cppcheck → 0 violations [3/6] SIL test suite .............. pytest --target=sil test_safe_state_on_busoff ... REQ-0042 → PASS (measured: 8.2ms) test_e2e_crc_error .......... REQ-0045 → PASS test_watchdog_timeout ....... REQ-0050 → PASS (measured: 18.4ms) coverage: 94.2% branch / 87.1% MC/DC [4/6] PIL test suite .............. pytest --target=pil test_safe_state_on_busoff ... REQ-0042 → PASS (measured: 9.1ms) test_watchdog_timeout ....... REQ-0050 → PASS (measured: 19.2ms) SIL vs PIL delta ............ +0.9ms → WITHIN BUDGET [5/6] HIL test suite .............. pytest --target=hil test_safe_state_on_busoff ... REQ-0042 → PASS (measured: 9.8ms) test_7ecu_busload ........... REQ-0060 → PASS (bus load: 61.4%) test_fault_injection_relay .. REQ-0055 → PASS [6/6] Traceability merge .......... merge_xil_results.py coverage_report.html ........ 47/50 requirements → COVERED 3 requirements not yet covered: REQ-0071, REQ-0072, REQ-0073 ────────────────────────────────────────────────────────────── PIPELINE: PASS ¦ Elapsed: 14m 22s ¦ PR merge unblocked ──────────────────────────────────────────────────────────────

## 4.1  What a Failure Looks Like

git push origin feature/watchdog-interval-change [3/6] SIL test suite .............. pytest --target=sil test_watchdog_timeout ....... REQ-0050 → FAIL AssertionError: measured 24.3ms, requirement: ≤20ms Assumption violated: "watchdog granularity 1ms on target OS" Requirement source: SW-REQ-0050 ← TSC-REQ-012 ← HARA-SG-003 ────────────────────────────────────────────────────────────── PIPELINE: FAIL ¦ PR merge BLOCKED Deviation traced to: HARA Safety Goal SG-003 Cost to fix now: ~€500 Cost to fix at system integration: ~€50,000 ──────────────────────────────────────────────────────────────

The pipeline does not just say FAIL. It tells you which requirement, which safety goal, and which HARA assumption is violated — and it traces the deviation back to its origin in the assumption chain.

# 5. Artifact Map — What the Pipeline Produces

Every stage produces artifacts in vendor-neutral formats that feed directly into commercial tools without manual conversion.

| Stage | Our Artifact | Format | Produced By | Imports Into |
| --- | --- | --- | --- | --- |
| L0 | requirements.reqif | ReqIF 1.2 XML | doorstop | Polarion ALM, IBM DOORS, Codebeamer, Jama |
| L0 | coverage_report.html | HTML + JUnit XML | merge_xil_results.py | Polarion Test, HP ALM, Jenkins dashboard |
| L1 | TaktflowSystem.arxml | AUTOSAR 4.x ARXML | arxmlgen + DBC | DaVinci Configurator, EB tresos — VALIDATED |
| L1 | taktflow.dbc | DBC / FIBEX | signal database | CANdb++, CANoe, CANalyzer, INCA, CANape |
| L2 | CanIf_Cfg.c / Com_Cfg.c | AUTOSAR C source | arxmlgen codegen | Any AUTOSAR BSW stack, MISRA checker |
| L3 | results_sil.xml | JUnit XML | pytest --target=sil | Jenkins, GitHub Actions, Polarion, IBM RQM |
| L3 | lcov.xml + MC/DC report | LCOV / LLVM format | gcov / llvm-cov | SonarQube, Squish Coco, LDRA Testbed |
| L4 | results_pil.xml | JUnit XML | pytest --target=pil | Same as SIL — identical format, comparable data |
| L5 | results_hil.xml | JUnit XML | pytest --target=hil | Same as SIL/PIL — unified in merge step |
| L5 | candump_YYYYMMDD.asc | CANoe .asc log | python-can recorder | CANoe, CANalyzer, Vector CANdb++ replay |
| All | trace_matrix.xml | ATML / custom XML | gen-traceability.sh | Polarion, Codebeamer — after ATML normalisation |
| All | taktflow.a2l (planned) | ASAM MCD-2 MC | pyelftools + pya2ldb | CANape, INCA, ATI Vision, pyxcp |

# 6. The Business Proposition

## 6.1  What This Is Not

- Not a replacement for your existing tools — DaVinci, CANoe, Polarion, DOORS remain unchanged

- Not a new process methodology — the V-model stays intact — we automate its execution

- Not a full-time headcount — a 4-week pilot produces a working pipeline on your existing project

- Not a toolchain migration — every artifact is in a format your tools already accept

## 6.2  What This Is

- The automation layer between your tools — the arrows that currently happen in meetings and shared drives

- A continuous verifier of the assumption chain — every commit tests whether reality still matches what was written above it

- An early warning system for expensive surprises — surfaces week-30 discoveries in week 2 at 1% of the correction cost

- An audit accelerator — traceability evidence is generated on every build — audit prep becomes a report not a manual exercise

## 6.3  The Pilot

One ECU project. One existing DBC. Your current manual test process running in parallel.

| Week | Work | Deliverable |
| --- | --- | --- |
| Week 1 | Discovery: DBC, existing test cases, target hardware, safety requirements baseline | NDA + access. Scope doc signed. |
| Week 2 | Pipeline stand-up: ARXML generation, SIL Docker environment, pytest restbus baseline | SIL green. First CI pipeline running. |
| Week 3 | PIL on your target hardware. HIL wired to your bench. First assumption deviations surfaced. | PIL + HIL running. First discrepancy report. |
| Week 4 | Traceability chain wired. Coverage report generated. Handoff documentation written. | Full pipeline. Traceability report. Runbook. |

| Fixed price: €8,000–10,000 4 weeks. Defined scope. No surprises. | Zero disruption to existing tools CANoe, DaVinci, Polarion all stay exactly as they are. | Retainer path optional 2 days/month keeps the pipeline maintained and growing. |
| --- | --- | --- |

## 6.4  The One-Line Pitch

"Every layer of automotive development carries forward assumptions about the layer below. We make those assumptions continuously testable. You find the expensive surprises in week 2 — not week 40."

Document v1.0  |  March 2026  |  Taktflow Engineering