TAKTFLOW SDV PLATFORM SysML · Capella · ReqIF Technical Reference, Comparison Analysis & Integration Guide Scope System Modeling + Requirements; Tools Covered Capella · SysML · ReqIF · doorstop; vs Commercial Polarion · DOORS · Rhapsody; Gap Priority P1 — ReqIF · P3 — Capella.

# Part 1 — SysML and Capella MBSE

## 1.1  What is SysML?

SysML (Systems Modeling Language) is an OMG standard for model-based systems engineering (MBSE). It is a profile of UML adapted for systems engineering — covering both hardware and software, not just software. It provides nine diagram types that together describe a system from requirements through behavior through structure.

| Diagram Type | Purpose | What it captures for your HIL |
| --- | --- | --- |
| Block Definition | System structure | ECU-1 through ECU-7 as typed blocks with ports and interfaces |
| Internal Block | Component wiring | CAN signal connections between ECUs, I/O lines to relay board |
| Requirement | Requirement text | ASIL D requirements with derivation and traceability links |
| Use Case | Operational scenarios | Fault injection scenario, safe state propagation, restbus test |
| Sequence | Temporal interactions | CAN message sequence during ECU boot + safe state trigger |
| State Machine | Behavioral states | SafeState FSM: NOMINAL → FAULT → SAFE → RECOVERY |
| Activity | Data flow / logic | ASIL D voting logic flow between ECU-1 and ECU-4 |
| Parametric | Constraints/performance | CAN bus load ≤ 80%, safe state latency ≤ 10ms |
| Package | Model organization | Partitions model into Requirements, Architecture, Test packages |

## 1.2  What is Capella?

Capella is an open-source MBSE tool built on Eclipse, developed originally by Thales. It implements the ARCADIA methodology — a four-layer architecture decomposition that maps directly onto automotive system development. It is the most practical free tool for producing SysML-compatible models that feed downstream ARXML and requirements toolchains.

| ARCADIA Layer | Question Answered | Taktflow Application |
| --- | --- | --- |
| Operational Analysis | What does the system need to do? | Vehicle safe state management: operator, ECUs, diagnostics tester interact |
| System Analysis | What are the system boundaries? | Taktflow system block: 7 ECUs, CAN bus, HIL controller, test host |
| Logical Architecture | How is it decomposed? | ECU roles: gateway, body, sensor, safety monitor, vECU restbus nodes |
| Physical Architecture | What runs where? | Mapping: ASIL D logic on TMS570, restbus on Docker, I/O on STM32H7 |

## 1.3  How Capella Feeds the Taktflow Pipeline

The key value of Capella for your platform is not just documentation — it is that the logical architecture model is the authoritative source for interface definitions that currently must be hand-written in ARXML. Once the XMI export bridge is built, the pipeline becomes:

Capella Logical Architecture (component interfaces, signal flows) │ ▼ capella_to_arxml.py (Python XMI transformer — ~300 lines) ARXML PortInterfaces + SWC ports ← currently hand-authored │ ▼ arxmlgen engine (existing) CanIf_Cfg.c Com_Cfg.c PduR_Cfg.c RTE headers │ ▼ compile → SIL → PIL → HIL Tested ASIL D firmware

Before the XMI bridge exists, Capella still adds value as a living architecture document and as the formal input for requirements derivation. The bridge is Phase 4 work — the modeling can start in Phase 1.

## 1.4  Capella vs Commercial SysML Tools

| Capability | Capella (Free) | IBM Rhapsody (€8k/yr) | Simulink (€4k/yr) | Enterprise Architect (€1.5k) | Notes |
| --- | --- | --- | --- | --- | --- |
| ARCADIA/MBSE methodology | ✓ Native | ✗ | ✗ | ~ Partial | Capella is purpose-built for ARCADIA |
| SysML XMI export | ✓ Full | ✓ Full | ~ Partial | ✓ Full | All export to XMI; Capella quality is high |
| ARXML import/export | ✗ Gap | ✓ Full | ✓ Full | ~ Partial | Rhapsody has native AUTOSAR bridge |
| Simulink model import | ~ Partial | ✓ Full | ✓ Native | ~ Partial | Simulink imports SysML XMI from Capella |
| Requirements traceability | ✓ Full | ✓ Full | ~ Partial | ✓ Full | Capella links model elements to requirements |
| ReqIF import/export | ✓ Full | ✓ Full | ✗ | ✓ Full | Capella has built-in ReqIF support |
| Eclipse plugin ecosystem | ✓ Rich | ~ Limited | ✗ | ~ Limited | Capella has 50+ add-ons on Eclipse |
| Learning curve | Medium | Very High | High | Medium | Capella docs are excellent |
| Active community | ✓ Strong | ~ Vendor-only | ✓ Large | ✓ Strong | Capella: Eclipse Foundation backed |
| License cost | Free | ~€8,000/yr | ~€4,000/yr | ~€1,500 | Clear winner on cost |

## 1.5  What the Capella Model Produces for Taktflow

### Artifacts exported from Capella:

- SysML XMI — importable into Simulink (Model-Based Design), IBM Rhapsody, Enterprise Architect

- Component interface definitions — each ECU's provided/required ports with signal names and data types

- Physical architecture diagram — ECU-to-hardware mapping — which software component runs on which physical node

- Requirements derivation — system requirements → component requirements → test cases, all linked in the model

- ReqIF export — Capella's built-in ReqIF export feeds directly into your requirements pipeline

### Python XMI transformer (the bridge you write):

# capella_to_arxml.py — ~300 lines import xml.etree.ElementTree as ET def extract_interfaces(capella_xmi): tree = ET.parse(capella_xmi) # find all LogicalComponent/ComponentPort elements # map to AUTOSAR PortInterface + SWC ports # emit as ARXML fragment for arxmlgen to consume for component in tree.findall(".//logicalComponent"): for port in component.findall("ownedFeatures[@xsi:type='cs:Component Port']"): yield build_arxml_port_interface(component, port)

## 1.6  Gap Analysis — Capella / SysML

| ID | Gap | Priority | Fix | Effort | Vendor Portability |
| --- | --- | --- | --- | --- | --- |
| C-01 | No Capella project exists | P3 | Create Capella workspace with 4-layer ARCADIA model for 7-ECU topology. Export SysML XMI. | 3–4 weeks | SysML XMI → Simulink, Rhapsody, EA |
| C-02 | No XMI → ARXML bridge | P3 | Write capella_to_arxml.py: parse Capella XMI, extract component ports, emit ARXML PortInterface fragments for arxmlgen. | 2–3 weeks | ARXML 4.x → DaVinci, EB tresos |
| C-03 | Architecture not in version control | P3 | Commit Capella .melodymodeller files to git. Add CI step to validate XMI well-formedness on every push. | 2 days | N/A — internal process |
| C-04 | No physical architecture mapping | P4 | Model ECU-to-hardware allocation in Capella Physical Architecture layer. Derives deployment ARXML (ECUInstance, FlatMap). | 1–2 weeks | ARXML ECU extract → DaVinci |

# Part 2 — ReqIF Requirements

## 2.1  What is ReqIF?

ReqIF (Requirements Interchange Format) is an OMG standard (ISO 26531) for exchanging requirements between tools. It is an XML format that every major requirements management tool — Polarion, IBM DOORS, Jama, Codebeamer, PTC Integrity — can import and export natively. It is the lingua franca of automotive requirements.

For ISO 26262 ASIL D, requirements traceability is not optional. The standard demands:

- System requirements → software requirements (derivation)

- Software requirements → software unit test cases (verification)

- Test case result → requirement coverage (evidence)

- All of the above in a format that a functional safety auditor can inspect

Without a ReqIF layer, your test pipeline — however technically excellent — cannot produce ISO 26262 Part 6 verification evidence. This is GAP-01: the highest-priority gap in the entire platform.

## 2.2  ReqIF File Structure

A ReqIF file is an XML document with a defined schema. The key elements for your use case:

<?xml version="1.0" encoding="UTF-8"?> <REQ-IF> <THE-HEADER> <REQ-IF-HEADER IDENTIFIER="taktflow-v1"> <CREATION-TIME>2026-03-01T00:00:00</CREATION-TIME> <TITLE>Taktflow ASIL D System Requirements</TITLE> </REQ-IF-HEADER> </THE-HEADER> <CORE-CONTENT> <SPEC-OBJECTS> <!-- Each requirement is a SPEC-OBJECT --> <SPEC-OBJECT IDENTIFIER="REQ-0042"> <VALUES> <ATTRIBUTE-VALUE-STRING THE-VALUE="REQ-0042"> <DEFINITION><ATTRIBUTE-DEFINITION-STRING-REF>ID </ATTRIBUTE-DEFINITION-STRING-REF></DEFINITION> </ATTRIBUTE-VALUE-STRING> <ATTRIBUTE-VALUE-STRING THE-VALUE="ASIL D"> <DEFINITION><ATTRIBUTE-DEFINITION-STRING-REF>ASIL </ATTRIBUTE-DEFINITION-STRING-REF></DEFINITION> </ATTRIBUTE-VALUE-STRING> <ATTRIBUTE-VALUE-STRING THE-VALUE="The ECU shall enter safe state within 10ms of detecting a CAN bus-off condition."> <DEFINITION><ATTRIBUTE-DEFINITION-STRING-REF>Text </ATTRIBUTE-DEFINITION-STRING-REF></DEFINITION> </ATTRIBUTE-VALUE-STRING> </VALUES> </SPEC-OBJECT> </SPEC-OBJECTS> <SPEC-RELATIONS> <!-- Links: requirement → test case, parent → child --> <SPEC-RELATION IDENTIFIER="REL-0042-TC"> <SOURCE><SPEC-OBJECT-REF>REQ-0042</SPEC-OBJECT-REF></SOURCE> <TARGET><SPEC-OBJECT-REF>TC-0042</SPEC-OBJECT-REF></TARGET> </SPEC-RELATION> </SPEC-RELATIONS> </CORE-CONTENT> </REQ-IF>

## 2.3  Two Implementation Paths

### Path A — doorstop (recommended for Taktflow)

doorstop stores requirements as YAML files in git. Every requirement is a plain text file with a unique ID. It integrates with CI, detects suspect links when source changes, and exports to ReqIF 1.2 for Polarion import.

# Setup pip install doorstop doorstop create SYS ./docs/requirements/sys # system reqs doorstop create SW ./docs/requirements/sw # software reqs # Add a requirement doorstop add SYS # Edits SYS-001.yml: # text: The ECU shall enter safe state within 10ms of bus-off. # links: [] # level: 1.0 # normative: true # Link SW req to system req doorstop link SW SW-001 SYS-001 # Export to ReqIF for Polarion doorstop export reqif ./docs/requirements/taktflow.reqif # Run in CI — detect broken links doorstop check # exit 1 if suspect links exist

### Path B — raw ReqIF Python generator

For teams that already have requirements in CSV/YAML and want full control over the output format. About 150 lines of Python produces a valid ReqIF 1.2 file that any tool can import.

# reqif_gen.py — generate ReqIF from CSV import csv, uuid from datetime import datetime def generate_reqif(csv_path, output_path): reqs = list(csv.DictReader(open(csv_path))) # ID, ASIL, Text, TestCase columns xml = reqif_header() for req in reqs: xml += spec_object(req["ID"], req["ASIL"], req["Text"]) for req in reqs: if req["TestCase"]: xml += spec_relation(req["ID"], req["TestCase"]) xml += reqif_footer() open(output_path, "w").write(xml)

## 2.4  Traceability Chain in pytest

The traceability link between requirements and test cases is established via a simple decorator or comment convention in pytest. Your existing gen-traceability.sh script parses these tags:

# tests/hil/test_safe_state.py import pytest # @requirement REQ-0042 # @asil D def test_safe_state_on_busoff(can_bus, hil_target): """ECU shall enter safe state within 10ms of CAN bus-off.""" can_bus.inject_busoff() response = can_bus.recv(timeout=0.015) decoded = hil_target.decode(response) assert decoded["SafeState"] == 1, "Safe state not set" assert decoded["FaultCode"] == 0x42, "Wrong fault code" # @requirement REQ-0043 # @asil D def test_safe_state_recovery(can_bus, hil_target): """ECU shall recover from safe state when bus restored.""" can_bus.inject_busoff() time.sleep(0.1) can_bus.restore_bus() response = can_bus.recv(timeout=0.5) assert hil_target.decode(response)["SafeState"] == 0

gen-traceability.sh then produces a matrix:

| Req ID | Requirement Text | ASIL | Test Case | Status |
| --- | --- | --- | --- | --- |
| REQ-0042 | The ECU shall enter safe state within 10ms of detecting a CAN bus-off condition. | D | test_safe_state_on_busoff | ✓ PASS |
| REQ-0043 | The ECU shall recover from safe state within 500ms when bus is restored. | D | test_safe_state_recovery | ✓ PASS |
| REQ-0044 | The ECU shall transmit DTC 0x42 on any safe state entry. | D | test_dtc_on_safe_state | ✓ PASS |
| REQ-0045 | The E2E protection shall detect single-bit errors within one message cycle. | D | test_e2e_single_bit_error | ✓ PASS |
| REQ-0050 | The watchdog shall trigger safe state if not serviced within 20ms. | D | test_watchdog_timeout | ⚠ Not written |

## 2.5  ReqIF Tool Comparison

| Tool | Cost | ReqIF Import | ReqIF Export | Git Native | CI Integration | Best For |
| --- | --- | --- | --- | --- | --- | --- |
| doorstop | Free | ~ Partial | ✓ Full | ✓ Native | ✓ Native | Open source teams, git-based workflow — recommended for Taktflow |
| IBM DOORS Next | €15k/yr | ✓ Full | ✓ Full | ✗ | ~ Via API | Large OEM/Tier-1 with existing DOORS infrastructure |
| Polarion ALM | €8k/yr | ✓ Full | ✓ Full | ✓ Git connector | ✓ Full | Mid-size Tier-1 — best commercial option for ReqIF-first workflow |
| Codebeamer | €5k/yr | ✓ Full | ✓ Full | ✓ Full | ✓ Full | Agile teams needing requirements + test management in one tool |
| Jama Connect | €6k/yr | ✓ Full | ✓ Full | ~ Limited | ✓ Full | Medical/automotive teams needing traceability audit trail |
| Raw ReqIF XML | Free | ✓ Full | ✓ Full | ✓ Manual | ✓ Full | Maximum portability — any tool that claims compliance must accept it |

## 2.6  Gap Analysis — ReqIF / Requirements

| ID | Gap | Priority | Fix | Effort | Blocks |
| --- | --- | --- | --- | --- | --- |
| R-01 | No requirements.reqif file exists | P1 | Create reqif/ folder. Define 50+ baseline ASIL D requirements. Use doorstop or raw ReqIF generator. Assign REQ-IDs. | 3–4 days | R-02, R-03, R-04 |
| R-02 | No @requirement tags in pytest | P1 | Add @requirement REQ-XXXX comments to all existing test cases. Update conftest.py to parse tags and emit to JUnit XML. | 2 days | R-03 |
| R-03 | gen-traceability.sh has no input | P1 | Wire gen-traceability.sh to parse @requirement tags from pytest output + reqif/ folder. Run in CI, fail if uncovered P1 reqs. | 2 days | R-04 |
| R-04 | No coverage report artifact | P2 | gen-traceability.sh should emit coverage_report.html: req ID → test case → pass/fail per xIL level. Upload as CI artifact. | 3 days | None |
| R-05 | Requirements not Polarion-importable | P2 | Validate reqif file against ReqIF 1.2 XSD. Test import into Polarion demo instance. Fix any schema issues. | 1 day | None |
| R-06 | No ASIL level attribute in ReqIF | P2 | Add ASIL attribute (QA, A, B, C, D) to ReqIF schema. Populate per requirement. Polarion ALM reads this natively. | 1 day | None |
| R-07 | No requirement derivation chain | P3 | Model SYS → SW derivation in doorstop. System requirements derive software requirements. Link both levels in ReqIF. | 1 week | None |

# Part 3 — How SysML, ReqIF, and ARXML Connect

## 3.1  The Full Artifact Flow

The three layers are not independent tools — they form a continuous chain where each layer feeds the next. Understanding this flow is the key to building a vendor-portable pipeline.

┌─────────────────────────────────────────────────────────────────────┐ │ CAPELLA MBSE MODEL │ │ Operational → System → Logical → Physical │ │ ├── Component decomposition (ECU-1..ECU-7 roles) │ │ ├── Interface definitions (CAN signals, port types) │ │ └── Requirements derivation (SYS → SW) │ └──────────────┬──────────────────────────┬───────────────────────────┘ │ SysML XMI export │ ReqIF export ▼ ▼ ┌──────────────────────┐ ┌───────────────────────────────────────┐ │ capella_to_arxml.py │ │ requirements.reqif │ │ XMI → ARXML bridge │ │ REQ-IDs + ASIL + derivation links │ └──────────┬───────────┘ └───────────────┬───────────────────────┘ │ PortInterfaces │ @requirement tags ▼ ▼ ┌──────────────────────┐ ┌───────────────────────────────────────┐ │ TaktflowSystem.arxml│ │ pytest test cases │ │ (strict PASS ✓) │ │ test_safe_state_on_busoff() │ └──────────┬───────────┘ └───────────────┬───────────────────────┘ │ arxmlgen │ pytest --target=sil/pil/hil ▼ ▼ ┌──────────────────────┐ ┌───────────────────────────────────────┐ │ BSW configs │ │ JUnit XML results │ │ CanIf Com PduR RTE │ │ + coverage_report.html │ └──────────────────────┘ │ REQ-0042 → PASS (SIL+PIL+HIL) │ └───────────────────────────────────────┘

## 3.2  Vendor Portability Map

Every artifact in the chain is in a format that commercial tools can consume natively. This is the "vendor-neutral boundary" principle — you build open-source, but any vendor tool can be plugged in at any layer.

| Layer | Our Artifact | Format | Imports Into |
| --- | --- | --- | --- |
| System Model | Capella .melodymodeller | SysML XMI | Simulink (model import), IBM Rhapsody, Enterprise Architect, MagicDraw |
| Requirements | requirements.reqif | ReqIF 1.2 XML | Polarion ALM, IBM DOORS Next, Codebeamer, Jama Connect, PTC Integrity |
| SW Architecture | TaktflowSystem.arxml | AUTOSAR 4.x ARXML | DaVinci Configurator, EB tresos, Vector AUTOSAR Builder — VALIDATED |
| Signal DB | taktflow.dbc | DBC / FIBEX | CANdb++, CANoe, CANalyzer, INCA, CANape — industry standard |
| Test Results | results.xml | JUnit XML | Jenkins, GitHub Actions, Polarion ALM, IBM RQM, HP ALM |
| Coverage | lcov.xml + MC/DC | LCOV / gcov XML | SonarQube, Squish Coco, Tessy, LDRA Testbed |
| Calibration (planned) | taktflow.a2l | ASAM MCD-2 MC | CANape, INCA, ATI Vision, pyxcp — any ASAM-compliant tool |
| Trace artifacts | trace.xml | custom + ATML | HP ALM, codebeamer, Polarion — after ATML normalization |

## 3.3  Full Commercial Comparison

The table below maps the complete Taktflow model/requirements stack against the equivalent commercial toolchain cost for a small Tier-1 supplier.

| Capability | Commercial Tool / Cost | Taktflow Equivalent | Gap / Notes |
| --- | --- | --- | --- |
| MBSE / System Modeling | IBM Rhapsody ~€8k/yr | Capella (free) | ✓ No gap — Capella produces equivalent SysML XMI. ARCADIA methodology is arguably more rigorous than Rhapsody for systems work. |
| Requirements management | Polarion ALM ~€8k/yr | doorstop + ReqIF (free) | ~ Minor gap — doorstop lacks GUI and advanced change management. ReqIF export is fully compatible with Polarion import. |
| ARXML authoring | DaVinci Dev ~€15k/yr | arxmlgen (custom, free) | ✓ No gap — ARXML passes strict schema validation. DaVinci import confirmed. |
| Req → test traceability | Polarion Test + DOORS | pytest + doorstop + ReqIF | ~ Gap currently — becomes full coverage after R-01..R-04 closed (1 week effort). |
| Model → ARXML bridge | DaVinci + Rhapsody bridge | capella_to_arxml.py | ✗ Gap — bridge does not yet exist. 2–3 week development effort. This is the key Phase 4 item. |
| Requirements review/approve | Polarion workflow engine | GitHub PR review | ~ Partial — GitHub PRs cover peer review. No formal approval workflow with role-based sign-off. |
| Bidirectional traceability | Polarion + DOORS round-trip | ReqIF + doorstop links | ~ Partial — forward traceability (req→test) works. Backward (test coverage→req gaps) needs R-03 closed. |
| Formal req validation | DOORS DXL scripts | suspect-links.py (exists) | ✓ Covered — suspect-links.py detects broken traceability. Equivalent to basic DOORS validation scripts. |
| Physical arch modeling | Rhapsody + UML deploy | Capella Physical layer | ~ Gap — Capella physical architecture layer planned (C-04). Covers same capability once modeled. |
| SysML XMI → Simulink | Rhapsody → Simulink | Capella XMI → Simulink | ✓ Covered — Simulink imports SysML XMI natively. Capella XMI quality is Simulink-compatible. |
| TOTAL COST (modeling+reqs) | ~€31,000/yr commercial | ~€0/yr (open source) | 10–15% capability gap after all gaps closed. For 85% of workflows, full functional parity. |

# Part 4 — Consolidated Gap Register

## 4.1  All Gaps by Priority

| ID | Area | Gap Title | Priority | Effort | Depends On | Fix Summary |
| --- | --- | --- | --- | --- | --- | --- |
| R-01 | ReqIF | Create requirements.reqif baseline | P1 | 3–4 days | None | doorstop create SYS; add 50+ requirements; export to ReqIF 1.2 |
| R-02 | ReqIF | Add @requirement tags to pytest | P1 | 2 days | R-01 | Comment convention in test files; conftest.py parses tags |
| R-03 | ReqIF | Wire gen-traceability.sh | P1 | 2 days | R-01, R-02 | Parse tags from pytest + reqif; emit coverage_report.html in CI |
| R-04 | ReqIF | Coverage report CI artifact | P2 | 3 days | R-03 | HTML matrix: req ID → test → SIL/PIL/HIL pass/fail result |
| R-05 | ReqIF | ReqIF schema validation | P2 | 1 day | R-01 | Validate against ReqIF 1.2 XSD; test Polarion import |
| R-06 | ReqIF | ASIL attribute in ReqIF | P2 | 1 day | R-01 | Add ASIL field to schema; populate per req |
| R-07 | ReqIF | SYS → SW derivation chain | P3 | 1 week | R-01 | Model two-level derivation in doorstop; export to ReqIF |
| C-01 | Capella | Create Capella ARCADIA model | P3 | 3–4 weeks | None | 4-layer ARCADIA for 7-ECU topology; export SysML XMI |
| C-02 | Capella | XMI → ARXML bridge | P3 | 2–3 weeks | C-01 | capella_to_arxml.py; extract ports; feed arxmlgen |
| C-03 | Capella | Capella in version control | P3 | 2 days | C-01 | Git .melodymodeller; CI XMI well-formedness check |
| C-04 | Capella | Physical architecture mapping | P4 | 1–2 weeks | C-01, C-02 | ARXML ECU extract from Capella deployment model |

## 4.2  Closure Sequence

The recommended closure sequence respects both priority and dependencies:

| Week | Actions | Deliverable | Unlocks |
| --- | --- | --- | --- |
| Week 1 | R-01: Create reqif/ + 50 requirements R-02: Add @requirement tags to all tests | requirements.reqif v1.0 Test tags in all test files | R-03, R-03, traceability CI |
| Week 2 | R-03: Wire gen-traceability.sh R-04: Coverage report in CI | coverage_report.html per commit CI fails on uncovered P1 reqs | R-05, R-06 |
| Week 3 | R-05: ReqIF schema validation R-06: ASIL attribute R-07: Start derivation chain | Polarion-importable ReqIF ASIL-tagged requirements | Portfolio demo, vendor conversations |
| Week 4+ | C-01: Begin Capella ARCADIA model (parallel with other Phase 2 gaps) | Capella project in git SysML XMI export | C-02, C-03, C-04 (Phase 4) |

# Part 5 — Implementation Guide

## 5.1  ReqIF — Getting Started (Week 1)

### Step 1: Install doorstop and create the repository structure

pip install doorstop # Create two-level requirement hierarchy doorstop create SYS ./reqif/sys --sep - # SYS-001, SYS-002... doorstop create SW ./reqif/sw --sep - # SW-001, SW-002... doorstop create TC ./reqif/tc --sep - # TC-001, TC-002... # Link levels: SW derives from SYS, TC verifies SW doorstop link SW SW-001 SYS-001 doorstop link TC TC-001 SW-001

### Step 2: Define baseline ASIL D requirements (SYS-001.yml)

active: true derived: false level: 1.0 links: [] normative: true ref: "" reviewed: null text: ¦ The ECU shall enter safe state within 10ms of detecting a CAN bus-off condition. header: Safe State on Bus-Off asil: D verification_method: test

### Step 3: Export to ReqIF for Polarion

# Export doorstop export reqif ./reqif/taktflow.reqif # Validate against ReqIF 1.2 XSD python tools/ci/validate_reqif.py ./reqif/taktflow.reqif # Add to GitHub Actions # .github/workflows/quality.yml - name: Validate ReqIF run: python tools/ci/validate_reqif.py reqif/taktflow.reqif - name: Check traceability run: doorstop check - name: Generate coverage report run: bash scripts/build/gen-traceability.sh

## 5.2  Capella — Getting Started

### Installation

# Download Capella from Eclipse Foundation (free) # https://www.eclipse.org/capella/download.html # Version: Capella 7.0 (latest stable) # Recommended add-ons: # - PVMT (Property Value Management Tool) — add custom attributes # - Capella XHTML Export — documentation generation # - MS Word Rich Text Export — for stakeholder reports # - Team for Capella — multi-user access (optional)

### Initial model structure for Taktflow

Taktflow.melodymodeller ├── Operational Analysis │ ├── Actors: ECU_Operator, DiagnosticsTester, VehicleNetwork │ └── Activities: Monitor_SafeState, Inject_Fault, Read_DTC │ ├── System Analysis │ ├── Taktflow_System (boundary) │ └── System_Functions: CAN_Communication, SafeState_Management, │ FaultDetection, E2E_Protection │ ├── Logical Architecture │ ├── Gateway_ECU — provides: CAN_Routing, NetworkManagement │ ├── Body_ECU — provides: ActuatorControl │ ├── Sensor_ECU — provides: SensorAcquisition │ ├── Safety_ECU — provides: WatchdogSupervision, DEM │ └── vECU_[5-7] — restbus simulation nodes │ └── Physical Architecture ├── TMS570LS3137 ← Gateway_ECU + Safety_ECU ├── STM32H743 ×2 ← Body_ECU + Sensor_ECU └── Docker_x86 ×3 ← vECU_5, vECU_6, vECU_7

## 5.3  Connecting the Three Layers in CI

# .github/workflows/full_pipeline.yml name: Full SDV Pipeline on: [push] jobs: arxml_validation: runs-on: ubuntu-latest steps: - uses: actions/checkout@v4 - run: pip install autosar-data armodel - run: python tools/ci/validate_arxml.py arxml/TaktflowSystem.arxml requirements_check: runs-on: ubuntu-latest steps: - run: pip install doorstop - run: doorstop check # fail on suspect links - run: doorstop export reqif reqif/taktflow.reqif - run: python tools/ci/validate_reqif.py reqif/taktflow.reqif sil_test: needs: [arxml_validation, requirements_check] runs-on: ubuntu-latest steps: - run: bash scripts/deploy/vecu-start.sh - run: pytest tests/ --target=sil --junit-xml=results_sil.xml - run: bash scripts/build/gen-traceability.sh - uses: actions/upload-artifact@v4 with: {name: coverage-report, path: docs/coverage_report.html}

Document v1.0  |  March 2026  |  Taktflow SDV Platform